To back up disk image of Rpi sdcard (boot OS)

Insert sdcard into a USB sdcard adapter

Use Win32DiskImager, choose H:\ device (make it the drive letter with sdcard)

Choose the storage location and image name in the Image File box

Click “Read” to read the image from the sdcard

It takes around 26 minutes to read the contents for a slower sdcard in USB adapter

To restore this image to a new sdcard for Rpi

Insert sdcard into a USB sdcard adapter

Use Raspberry Pi Imager

Choose the correct device (Rpi Zero 2W)

Scroll down to Custom for image to use, select the .IMG file created using Win32DiskImager above

Select the H:\ device (make it the drive letter with sdcard)

Running Rpi for first time after inserting new sdcard

Turn on hotspot, let it connect to network, find IP, use Terminal to ssh

Actually, it took a little while but it was already running rpi-connect and signed in…might need to power it on/off more than once and just be patient while watching the hotspot to make sure it connects.