Install Raspberry Pi OS, 64-bit for Pi4B or Pi Zero W2

SSH to rpi (username@ip.ad.dr.ess) then enter password

Follow instructions to install rpi-connect at: https://www.raspberrypi.com/documentation/services/connect.html#install-connect

This works fine, but I think connecting over SSH will be just fine, and rpi-connect allows an SSH connection as well as screen sharing.

rpi-connect on

Use this command to enable it, probably should go into the bootup script

The Pi Zero W2 seems to draw 0.12 to 0.15A just sitting at idle, will need to see what it draws when connected to the tcam-mini

Making it automatically start rpi-connect on:

Key steps:

- Enable auto-login:

  - Access the Raspberry Pi configuration tool (raspi-config).

  - Navigate to "System Options" > "Boot/Auto Login".

  - Select "Desktop Autologin" and choose the desired user (usually "pi").

- Create a startup script:

  - Using /etc/rc.local:

    - Open the /etc/rc.local file with a text editor (e.g., sudo nano /etc/rc.local)

    - Add the line: rpi-connect & (the ampersand ensures the command runs in the background)

    - Save the file and reboot your Raspberry Pi.

I ran sudo raspi-config

System Options>Boot Auto Login> select option B2, text console autologin

- I tried the B4 desktop autologin GUI option, but it draws > 2x more power when running, so just use the text console autologin

Power saving stuff:

Sudo raspi-config

2. Display Options>D4 Composite turns off HDMI, not sure if it saves much, but maybe 20mA (0.02A)

Sudo rfkill block 0                   this blocks Bluetooth, but I don’t see a big difference in power draw