This is a tutorial to collect TCam image and save on ResPi.

# Install system on ResPi

- Following the Respberry Pi official webpage’s instructions to flash system.

- Entering ResPi system

  - Turn on SPI, UART using command ‘sudo raspi-config’

  - sudo nano cmdline.txt to edit the file from the path below:

  - Setting ‘spidev.bufsiz=65536’ in path /boot/firmware/cmdline.txt.  Reminder: add this sentence in the same line!

# Python code

Check the examples in github: https://github.com/danjulio/tCam/tree/main/python

Two py files in the folder ‘./python code/’ is based on these examples.

Run the code command: ‘python3 tcam_tjsn.py’

## No hang up command:

nohup python /home/lab/tcam/python/examples/tcam_tjsn.py

## Automatically running after reboot

open terminal: sudo crontab -e

add new line: @reboot python /home/lab/tcam/python/examples/tcam_tjsn.py

# SSH login

The device was connected to eduromo wifi

Ensuring your device is connect to the same WIFI.

Login command: ‘ssh lab@10.138.26.101’

Logout command: ‘exit’